Date :- Jan 25, 2005
PH – 804 (spring-2005)
Data for Expt No. – 2 Single Stage CE Amplifier

	Sr.No
	Roll No.
	(

	Common Emitter Amplifier

	
	
	
	Vcc

(Volts)
	AV
	RO
(k()
	S

	1
	04412001
	208
	9
	100
	1.6
	4

	2
	04412002
	213
	10
	120
	1.8
	5

	3
	04412003
	234
	12
	140
	2.2
	6

	4
	04412005
	243
	9
	160
	2.7
	4

	5
	04412301
	247
	10
	100
	1.6
	5

	6
	04412701
	258
	12
	120
	1.8
	6

	7
	04412901
	264
	9
	140
	2.2
	4


NOTE:-  1) fmin (-3 db point for CE Amplifier) is 100 Hz.

                2) Gain (AV) is specified at 1 KHz.

                3)  
[image: image1.wmf]

 EMBED Equation.3  [image: image2.wmf]Rac

Rdc

Vcc

I

CQ

+

=


                                                                                                        Prof. B.P.Singh
                                                                                                        (CII- PH-804)

                                                                                                                                     Date:- January 25, 2005
PH-804(spring-2005)
Measurements for No. – 2:     Single stage CE Amplifier

A)  Single stage CE Amplifier without Feedback:-
1) DC Measurements:-

i)    Measure VB, VE, VC  w.r.t ground.

ii)   Determine VBE,VCE
iii)  Determine IC  (  VE / RE
    Report Operating Point Q (VCE, IC) & compare with the expected  

    value.

2) AC Measurements:-

Determine and compare the parameters( i to iii ) at mid-band frequency of 1 kHz. 

i)    Gain

ii)   Rin
iii)  Rout
iv)  Plot the Lower frequency response (1 kHz to 60 Hz)

v)   Determine f -3dB from the graph & compare with the expected

    value.

B) Single stage CE Amplifier with Feedback:- 

Only AC measurements:-

                        Determine the following parameters at mid-band frequency of 1 kHz.

i)   Gain

ii)  Rin
iii) Rout

------------------------------------------------------------------------------------------------------------

Note:- If the actual components value used differ from the theoretical values, recalculate   


the expected values of different parameters. Use these new values for comparison 

 
with experimental results.

 
Design should be ready before coming to the laboratory  for the experiment.

------------------------------------------------------------------------------------------------------------

_1104303830.unknown

_1104304047.unknown

