Instructions to start RH O2 runs
A) To start RH02 runs

1. Click on PL icon on the desktop. Select normal mode.

2. Small pico recorder will open.

3. Click on file . To change the settings click on new settings.

4. Mode is real time continuous. Then click OK

5. Sampling interval = 1Hr. (Time between two samples.)

6. Maximum no. of samples = Give huge no. Run will stop after collecting those many samples.

7. Reading per sample = As many as possible.(It will give average reading between the sampling interval.)

8. Then Click OK

9. Then it will show RHO2 and com port 1 selected.

10. Then click ok

11. Choose temperature , go to options and click on 

12. Similarly do for humidity.

13. Then it will display your setting in the recording window.

14. Click on file again. Go to new data. Give suitable file name (usually we are giving the date on which you are staring the run. eg 3oct05.plw.) File location has to be F/RPCMON/RHO2/**.plw 

15. Then say Ok 

16. Start the run by pressing red arrow in the recording window.

B) To stop the run

If you want to stop the run press black square in the recorder window.

Once the run is stopped save the file in .txt extension by using save as option in the file menu. If you give the same name it will overwrite the plw file. Preferably give different name.

C) To restart the run without changing the settings

1. Go to file menu. Click on new data. 
2. Give suitable name. 

      3.     Start the run by   pressing red arrow 

