STUDY OF COSMIC RAY MUON DETECTION USING SCINTILLATION COUNTER

AND 
WAVELENGTH SHIFTING FIBER
                  SUBMITED FOR
                     VSRP-2009

             UNDER GUIDANCE OF   

        PROF.SUDESHNA BANERJEE

                         BY  
       ARUNODAYA BHATTACHARYA
     (Msc(physics),university of delhi)

Motivation:
 Muon detectors are employed in many major experiments like the CMS experiment. In laboratory , the principal source of muons is cosmic rays.Although, there are many methods of detecting muons, but scintillators are widely used owing to their fast response and low cost. A large array of scintillators can give us the track of the muon event and by knowing the track of a charged particle we can determine its momentum and  also it is used to form trigger for various applications. For this it is important to have an idea of the position coordinate of a particular event. This feat is achieved more or less accurately by the Resistive Plate Chambers as it gives the precise measurement of the track of the events,hence,muon event can be localised.But what we are trying to study is  the possibility that a muon event can be localized using scintillator paddle also.  
Abstract: In order to know the position of muon event on the scintilator we intend to use four wavelength shifting fibers grooved into a scintillator tile and all four fibers are connected to four photomultiplier tubes. So when an event takes place, we study the light of each of the fibers and infer about the position of the event. In order to ensure that the event in the scintillator is a genuine event we use RPC to trigger the scintillator. Moreover, RPC data gives a precise measurement of the event position, so we can co-relate the RPC data with the scintillator data. Jus to make sure that our paddle system works we initially connect all four fibers to a single PMT.
Principle of Operation of Scintillators:

The scintillator detector working principle is based on the detection of the light produced by the charged particle traversing the scintillator(1) and scintillator is a material that converts the incident particle energy into a detectable light. There are many varieties of scintillators available viz. inorganic,organic,liquid etc. For muon detection we are employing organic(acrylic) scintillaotrs as their response is fast.
To be a good scintillator , a material should convert as large a fraction as possible of the incident radiation energy to prompt fluorescence,while minimizing the generally undesirable contributions of phosphorescence and delayed fluorescence.

Here,the light that can contribute to an output pulse is generally limited to prompt fluorescence asthe time constants of the measurement circuits are set much smaller than typical phosphorescence and delayed fluorescence decay time.

A paddle has been made using the plastic scintillator and the materials used and the constructional details are given later.

Resistive Plate Chambers:
[image: image30.emf]268K


29.78V


R8


1


198.9K


R9


0.01uF


6


4.7nF


Dy7


199.8K


0.01uF


R10


29.50V


8


Dy8


199.7K


17


492.6V


R1


7


Dy9


R11


198.8K


29.81V


16


Dy10


R12


198.5K


2


0.01uF


Dy11


C5


15


Dy12


39.75V


+HT


R13


198.6K


29.75V


SIGNAL


R2


R14


Focussing


electrode


198.4k


29.77V


3


Dy1


R15


Cathode


198.5k


29.89V


4.7nF


Dy2


0.01uF


R16


198.7K


29.75V


R3


Dy3


14


C1


Anode


249K


29.81V


R4


Dy4


4


C2


C6


1K


29.76V


R5


13


C3


50k


820K


10


29.72V


R6


5


Dy5


C4


199K


Bleeder resistance=3.32M Ohms


29.71V


R7


12


Dy6


21




268K

29.78V

R8

1

198.9K

R9

0.01uF

6

4.7nF

Dy7

199.8K

0.01uF

R10

29.50V

8

Dy8

199.7K

17

492.6V

R1

7

Dy9

R11

198.8K

29.81V

16

Dy10

R12

198.5K

2

0.01uF

Dy11

C5

15

Dy12

39.75V

+HT

R13

198.6K

29.75V

SIGNAL

R2 R14

Focussing

electrode

198.4k

29.77V

3

Dy1

R15

Cathode

198.5k

29.89V

4.7nF

Dy2

0.01uF

R16

198.7K

29.75V

R3

Dy3

14

C1

Anode

249K

29.81V

R4

Dy4

4

C2

C6

1K

29.76V

R5

13

C3

50k

820K

10

29.72V

R6

5

Dy5

C4

199K

Bleeder resistance=3.32M Ohms

29.71V

R7

12

Dy6

21

Fig.1
The Resistive Plate Chamber (RPC) is a gas - based detector with good spatial as well as timing resolution(3).. It consists of  two parallel electrodes made up of float glass with a volume resistivity of about 10^12Ω-cm. The two electrodes, 2mm thick, are mounted 2mm apart by means of highly insulated spacers. A suitable gas mixture is flown at the atmospheric pressure through the gap while an appropriate electric field is applied across the glass electrodes through a resistive coating on their outer surfaces. An ionizing charged particle traversing the gap initiates an avalanche in the gas volume that results in a local discharge of the electrodes. This discharge is limited to a tiny area of about 0.1cm2 due to the high resistivity of the glass electrodes and the quenching characteristics of the gas. The discharge induces an electrical signal on external pickup strips on both sides orthogonal to each other, which can be used to record the location 
and time of ionization. The discharge area recharges slowly through the high resistivity glass plates and the recovery time is about 2secs.

[image: image1.emf]Equivalent circuit of RPC: [image: image2.emf]Fig.2
Recovery time:
  Where   R=resistance of glass

                C=effective capacitance between the plates

                A=area of glass plates

                εo = absolute permit 
                κ = dielectric constant

Voltage-current characteristics of RPC:
· Dimensions of RPC= 1m*1m

· Thickness of glass electrode=3mm

· Gap between two glass plates=2mm

· Width of pick up strips=2.8cm

· Flow rate of gas=30cc/min

· Flow rate of gas in 1 RPC=2.5cc/min

· Temperature=27.64 deg.Cel.

· Humidity=48.66%

[image: image3.emf]   [image: image4.jpg]Applied voltage (V)

1600

1400

1200

1000

800

600

400

200

V-l Characteristic of RPC

Current (nA)





[image: image5.jpg]Negative voltage(V)

1600

1400

V-l Characteristics of RPC

1200

1000

800

600

[
i
i
/

400

200

0

1000 2000 3000 4000
Current (nA)

5000

6000





 Plateauing of the Paddle:
In order to proceed with our aim, firstly, we should know how to plateau the paddle(i.e. the PMT). The paddle to be plateaud is alignment over another paddle of same dimension. Now a second paddle of smaller dimension is kept at a distance of 23.5cm from the bottom paddle. Over this paddle , a narrow paddle(dimensions given below) is aligned so that paddle 1 and 2 are in contact with each other . The output of paddle 1,2 and 4 are discriminated and ANDed to form a 3F coincidence. The output of the main paddle is deliberately delayed and ANDed with the 3F signal(i.e. 4F). Here,3F act as a trigger trigger that ensures that the event is a genuine muon event and only when 3F is present, the main paddle signal will go to the scalar. This type of alignment is done keeping in mind that any event that generates 3F should also generate 4F. So,we are ensuring that the efficiency of the paddle is close to  100%[image: image6.emf]1


D3


S1


2


D2


3


D4


MAIN


PADDLE


4


Efficiency=(4F/3F)*100


1


16


15


2


1,2,3,4=PADDLES


3F


D=DISCRIMINATOR


4F


1


2


3


S2


S=SCALAR


DETERMINATION OF EFFICIENCY OF AN        


                EXISTING  PADDLE AND 


         OPERATING VOLTAGE OF PMT


S3


DELAY


D1




1

D3

S1

2

D2

3

D4

MAIN

PADDLE

4

Efficiency=(4F/3F)*100

1

16

15

2

1,2,3,4=PADDLES

3F

D=DISCRIMINATOR

4F

1

2

3 S2

S=SCALAR

DETERMINATION OF EFFICIENCY OF AN        

                EXISTING  PADDLE AND 

         OPERATING VOLTAGE OF PMT

S3

DELAY

D1

                                                       
Fig.3
                       EFFICIENCY=(4F/3F)*100 %
[image: image28.wmf]0

0

=

÷

ø

ö

ç

è

æ

÷

ø

ö

ç

è

æ

@

=

rke

ke

r

t

l

A

A

l

RC

 


        Fig.4    GEOMETRY ARRANGEMENT OF THE PADDLES

[image: image29.wmf]0

0

=

÷

ø

ö

ç

è

æ

÷

ø

ö

ç

è

æ

@

=

rke

ke

r

t

l

A

A

l

RC

 




No.of paddles used=4

· Dimensions:

· 1st paddle=20cm*2cm*1cm

· 2nd paddle=20cm*5cm*1cm

· (main)3rd paddle=30cm*20cm*1cm

· 4th paddle=30cm*20cm*1cm

· Preset time=15min
· Discriminator level= -30mV

	Applied voltage(v)
	Current (mA)
	3F counts
	4F counts
	Counts of main paddle
	Efficiency(%)
	Noise rate

	1100
	0.2
	254
	0
	2
	0
	0.0023

	1200
	0.2
	351
	4
	57
	1.4
	0.0634

	1300
	0.2
	388
	8
	491
	2.06
	0.5456

	1400
	0.2
	383
	80
	463
	2.89
	0.5145

	1500
	0.2
	333
	223
	18448
	66.70
	20.4979

	1600
	0.2
	349
	322
	33340
	92.30
	37.045

	1700
	0.2
	352
	343
	57990
	97.5
	64.434

	1800
	0.2
	376
	370
	88477
	98.4
	98.3078

	1900
	0.2
	402
	395
	128406
	98.3
	142.6756


[image: image7.emf]Variation of Efficiency and Noise Rate

0

20

40

60

80

100

120

1000 1200 1400 1600 1800 2000

applied voltage(V)

efficiency(%)

0

20

40

60

80

100

120

140

160

noise rate

Efficiency(%)

Noise rate


The operating voltage of PMT is chosen somewhere in the middle of the plateau. We choose the operating voltage to be 1650V.

A small understanding and overview the Photomultiplier tubes is given in the next section
Photomultiplier Tube:

It is a device that converts the extremely weak light output of a scintillation pulse into a corresponding electrical signal(2). 


 Fig5  Schematic of a photomultiplier tube coupled to a scintillator
Photomultipliers are constructed from a glass envelope with a high vacuum inside, which houses a photocathode, several dynodes, and an anode. Incident photons strike the photocathode material, which is present as a thin deposit on the entry window of the device, with electrons being produced as a consequence of the photoelectric effect. These electrons are directed by the focusing electrode towards the electron multiplier(consisting of a number of electrodes called dynodes) where electrons are multiplied by the process of secondary emission.

The Photomultiplier tubes typically utilize 1000 to 2000 volts to accelerate electrons within the chain of dynodes. Voltages are distributed to the dynodes by a resistive voltage divider known as BLEEDER CHAIN, (shown below). 

Base construction of the PMT::In order to understand PMT working in detail, a base has also been made  by employing the materials shown in the circuit diagram and their prescribed values.  The assembly of these components has been made on a bose holder  that gets fixed to the  bottom of a photomultiplier tube. A schematic representation of the circuit used is  given overleaf. All the materials shown in the circuit are soldered carefully on the base holder.

[image: image9]Fig6
Result of PMT base assembly:The base of the PMT has been made by employing the bleeder resistance circuit  as shown and voltages across different pins have been measured. The voltages across pins show stability and,thus, the circuit  is ready to use. The bleeder resistance measurement    (between pin 7 and pin21) has been done and it is satisfactory with the literature. 
Construction of the paddle:
As we know , to localize a muon on a scintillator we need to make a scintillator paddle and study it. A paddle consists of a scintillator tile with optical fibers engrooved in it and connected to  Photomultiplier tube. Initially,we use the four fiber assembly and all four fibers are connected to a single PMT. This would give an idea about how to proceed next and what kind of difficulties we may encounter with the actual experiment. 
· Materials used:
· Scintillator:

· BICRON BC404 (polyvinyltoluene)
· Refractive index= 1.58

· Green wavelength shifting fibers(single clad)
· BICRON BCF 91A

· Core material=Polystyrene

· Refractive index=1.60

· Cladding material= PMMA(polymethylmethacrylate)

· Refractivew index=1.49

· White fibres(single clad)
· Transparent cookie

· Photomultiplier tube (head-on type)

· Type=EMT 9807B (ten step)

 Constructional detail:
· A plastic scintillator of dimension 12”*12”*1” is taken.

· Four grooves are cut in the scintillator material.

· A wavelength shifting fibre is inserted into each groove.

· The other end of the fibre is spliced with a clear fibre using heat shrinkable tube. This is done as attenuation length of the clear fibre is less compared to that of wavelength fibre.

·  All the fibres are enclosed in black heat shrinkable tubes to avoid any external light pick up.
· All the four fibres are inserted into a cylindrical cookie and fibres are cut and polished so that their face is perpendicular to the axis of the cookie.

· Cookie is glued to the face of  Photomultiplier tube using thick black insulation tapes.

· The scintillator is wrapped carefully in tyvec paper used as reflector and then wrapped in tedlar sheets which prevent any accidental leakage of light from the scintillator.

· The entire setup is mounted on a 50”*12” wooden board to impart strength to the setup.

Due to time constraint ,this paddle has not been plateaud. Instead,the operational voltage of the PMT is obtained from the operation manual. 

Checking for light leakage:
High voltage (1650V) is appied to the PMT base and its output is studied in an oscilloscope. To study this we adjust the time scale in such a way tha we get a noise band along with a pulse.

Then we switch off the lights. If the band becomes narrower,then there is a light leakage. In our case, we didn’t encounter any problem of such type. A typical PMT pulse from four fiber paddle is shown in the next section.
Study of Individual Count rate of the Paddle:
The individual count rate (noise rate) of the paddle has been studied at different discriminator thresholds and it gives satisfactory results.

· Applied voltage=1700V
· Preset Time=15minutes

[image: image10.emf]DISCRI-


MINATOR


SCALAR


ANALOG


NIM


STUDY OF COUNT RATE OF THE PADDLE


PADDLE


H.V.




DISCRI-

MINATOR

SCALAR

ANALOG NIM

STUDY OF COUNT RATE OF THE PADDLE

PADDLE

H.V.


Fig.7
	Discriminator level
	Counts per 15 minutes
	Count rate

(per sec.)

	-23mV
	7663
	8.5

	-30mV
	2222
	2.5

	-40mV
	508
	0.56

	-50mV
	225
	0.25

	-60mV
	128
	0.14

	-70mV
	119
	0.1


The variation has been shown graphically. We expect the count rate to fall down as threshold voltage is increased as threshold voltage in discriminator is basically a voltage upto which the discriminator doesn’t give any output and output is present only when the input pulse amplitude is greater than the threshold voltage. This helps us in discriminating the signal from the random noise. 
                   ANALOG OUTPUT OF THE PADDLE
[image: image11.png]@ 20.0mve 1.00 V. S00mv < J[20.0ns 5.00GS/S @ -48.8mv
347 0.00000 5 10k points

Assign
save [ 10
waveform

Save Save Save Recall Recall

File
Screen Image| waveform setup waveform setup i

utili





Amplitude of PMT pulse= -48.8mV; Width of the pulse=8nsec.
[image: image12.emf]Count Rate Of The Paddle Constructred

0

1

2

3

4

5

6

7

8

9

-80 -70 -60 -50 -40 -30 -20 -10 0

Discriminator voltage(mV)

Count rate(per second)


Upto this part we have not used the trigger pulse from the RPC. In the next section we will study our paddle using 8F trigger from the RPC stack. This takes  us a step closer to our aim!!
Study of Muon pulses using RPC trigger:

There are 12 layers of RPC with 32 read-out channels ,each for x and y side. The output of these channels(strips)are used for trigger generation.We are using 8 RPCs to generate a trigger,what is known as an 8-FOLD trigger logic. To generate 8F,we “AND” the output of each RPC. 
[image: image13.emf]LAYER 00


1


PADDLE


GATE


A


D


C


RPC


18


2


TRIGGER


DISCRIMINATOR; -20mV THRESHOLD


18


2


18


2


8F


18


2


18


2


LAYER 01


DELAY


CAMAC BUS


LAYER 02


5.2nsec./m of cable


DELAY


LAYER 03


LAYER 04


1


2


3


4


5


6


7


8


9


LAYER 05


18


2


18


2


LAYER 06


18


2


LAYER 07


18


2


PC




LAYER 00

1

PADDLE

GATE

A

D

C

RPC

18 2

TRIGGER

DISCRIMINATOR; -20mV THRESHOLD

18 2

18 2

8F

18 2

18 2

LAYER 01

DELAY

CAMAC BUS

LAYER 02

5.2nsec./m of cable

DELAY

LAYER 03

LAYER 04 1

2

3

4

5

6

7

8

9

LAYER 05

18 2

18 2

LAYER 06

18 2

LAYER 07

18 2

PC


Fig.8
[image: image14.emf]17


RPC LAYER02


31


23


9


2


16


24


10


1


29


15


25


GEOMETRY ALIGNMENT OF THE SCINTILLATOR ON THE RPC LAYER02


11


0


28


14


26


0


12


27


13


27


13


26


12


14


25


11


1


15


28


24


10


2


16


29


23


X-STRIP NUMBER


3


17


30


22


9


4


31


21


8


5


18


20


7


6


19


SCINTILLATOR


6


7


20


5


19


21


Y-STRIP NUMBER


4


18


22


8


30


3




17

RPC LAYER02

31

23

9

2 16

24

10

1 29 15

25

GEOMETRY ALIGNMENT OF THE SCINTILLATOR ON THE RPC LAYER02

11

0 28 14

26

0

12

27 13

27

13

26 12

14

25 11

1

15

28

24 10

2

16

29

23

X-STRIP NUMBER

3

17

30

22 9

4

31

21 8

5

18

20 7

6

19

SCINTILLATOR

6

7

20

5 19

21

Y-STRIP NUMBER

4 18

22

8

30 3


Fig.9
Alignment of the paddle : In order to know the exact position of the event on the paddle ,we need to align it with the x-y strips of the RPC. RPC system would give the x-y position of the event on the strips and this data would serve as a reference to localize the muon event on the paddle. So we can correlate these two independent datas  and conclude our results.

      The paddle is aligned on the third RPC layer.

       X position of the paddle= between strip number 14 to 23

       Y position of the paddle= between  strip number 19 to 28

We desire to study the events passing through third RPC layer and if the muon has passed through the paddle also the paddle would give large signal. So we can have a correlation between the x-y coordinates of the event on the particular layer and the Paddle output.

So, we proceed to run the DAQ system of the RPC(for track and position determination of the muon event) and storage oscilloscope (for storing the amount of charge collected). But we faced a problem with the trigger rate of 8F trigger.

Trigger rate of 8F = 4Hz but the rate at which storage oscilloscope the data and store it is 2Hz only. So , somehow trigger rate needs to be redused or we use the ADC system. We decide to use the ADC system as done earlier. 8F trigger is fed as GATE for ADC and paddle pulse is fed as input to ADC. Rest everything is same as earlier including the delays. The DAQ system and ADC are run simultaneously in order to achieve a correlation between the two datas.

Just to make sure that we are getting a nicely resolved signal peak, we draw the histogram of the ADC output

The paddle is kept over the third RPC layer. The 8F and paddle signal is first analysed in an oscilloscope:
· Applied voltage to paddle=1850V
· Channel 1= 8F RPC trigger

Gate width=53.4nsec.

· Channel 2=Paddle signal

Gap between leading edge of the signals=178.3nsec.

[image: image15.jpg]-Gl

@ 500mv o

20.0mvVe

100ns
i+¥ 0.00000 s

5.00GS/S
10k points

@ ~ 370mV)





As we can see, there is a time delay between paddle signal and RPC trigger. It is because the trigger from the RPC arrive late as the signal propagates through a lot of electronics and each electronic module imparts a delay to the trigger ,thereby, making the trigger to arrive late with respect to the paddle pulse. To bring the PMT signal into coincidence,delaying of it necessary
In order to bring the paddle signal in coincidence with the RPC signal,we  delay the paddle signal using LEMO cables
1 m of Lemo cable =5.2 nsec. of delay

Length of cable used=38m

    Net delay=38*5.2=197.6nsec.
[image: image16.jpg]@ s00mve @ 10.0mve b 5.00GS/S
10K points

Assign
Save (10
‘waveform

Save Save Recall

File
Screen Image| waveform waveform 18)un 2009

utilities 13:13:01





The only problem encountered is the tremendous        attenuation of the PMT signal because of the long delay cables. (The amount of attenuation present makes the signal almost negligible!)

Now,the 8F signal is fed as GATE input to the ADC and PMT signal is given as input to the ADC. Whenever GATE input is present ,the ADC gives the output in the form of charge  of the analog signal. The data is stored in a text file using c++ codes and analysed using various softwares. The histogram of the ADC output is given as follows:
(The second plot is only the magnified scale plot of the first plot)
[image: image17.emf]Graph between ADC output and Signal and Pedestal counts

0

10000

20000

30000

40000

50000

60000

70000

80000

90000

0 5 10 15 20 25 30 35 40 45 50 55 60 65 70

ADC output

Freq. (Pedestal+paddle)

0

1000

2000

3000

4000

5000

6000

7000

8000

Pedestal

+paddle

Pedestal


[image: image18.emf]Graph between ADC output and Signal and Pedestal counts

0

1000

2000

3000

4000

5000

6000

7000

8000

0 5 10 15 20 25 30 35 40 45 50 55 60 65 70

ADC output

Count (Pedestal+paddle)

0

1000

2000

3000

4000

5000

6000

7000

8000

Count (Pedestal)

Pedestal+

paddle

Pedestal


Here ,the broad hump is the muon signal peak and sharp peak is the pedestal(offset of ADC) peak. In this plot, the pedestal value has been separately determined and plotted but as you can see, the pedestal plot does not coincide with the plot of pedestal+paddle. Ideally it should do so. This happened because input of the ADC was kept floating. Pedestal should be determined at same conditions at which the actual experiment is performed. So, pedestal can’t be determined by keeping the input signal connected and not giving high voltage to PMT! (Also pedestal depends on GATE WIDTH ). So in order to determine pedestal correctly, we have given high voltage to PMT( 1800V) and instead of gate from RPC we have used gate from a pulsar with same gate width but high frequency. Now as we know ,output will come only when gate is present, also since the frequency of the gate is high,the probability of PMT pulse being present with the gate is very less  and what we get is the pedestal distribution of the ADC.
[image: image19.emf]Graph between ADC output and Signal and Pedestal counts

0

10000

20000

30000

40000

50000

60000

70000

80000

90000

100000

10 15 20 25 30 35 40 45 50 55 60 65 70

ADC counts

freq.of pedestal+paddle

0

2000

4000

6000

8000

10000

12000

14000

freq. of pedestal

pedestal

+paddle

pedestal


As we can see here the pedestal distribution has almost overlapped with the pedestal distribution when  PMT signal was read out using  8F RPC signal as GATE. Still there is not a complete match and its reason we still don’t know.
As we can see from the plots that the paddle output is very weak “BECAUSE OF THE DELAYS” and most of the ADC is output is pedestal only. Sol we switch over to storage oscilloscope in which channel2 (RPC TRIGGER) is used to trigger Paddle pulse(channel 1) in the oscilloscope itself and it also calculates the pulse height of the paddle pulse (of the pulse present on the window) in .xls format. So ,no delay is required,and hence we avoided the attenuation.To calculate the charge present in the Paddle pulse,we calculate the area under the curve of the pulse.The circuit is same as fig.7,except for ADC we have storage oscilloscope.( Although , this is not a very accurate way of calculating the charge,but this gives an approximate idea. We are forced to adopt his method because of low signal amplitude of the PMT pulse.  )
· This part of the experiment confirms that RPC signal could be used to trigger our paddle efficiently. The only problem to solve is how to avoid the attenuation of the paddle signal. 
(( Moreover ,till now the DATA ACQUISITION SYSTEM of the RPC has not been used yet which gives the event position and the muon tracks. This is done later.))
· Channel 1=Paddle signal

· Channel2=RPC 8F trigger

In our case, we store the amplitude of the PMT pulse using the oscilloscope. The oscilloscope calculates the amplitude at fifty points in the scope-window. We collect a lot of events and redraw the pulse shape using the data to determine the window in which maximum pulses are concentrated. We perform our analysis in that window only. In this case the window has around 11350 stored events.

        [image: image20.emf]50OHM


R15


CRO


4.7nF


SIGNAL


BLEEDER


 CIRCUIT


C5


1K




50OHM R15

CRO

4.7nF

SIGNAL

BLEEDER

 CIRCUIT

C5

1K


               Fig.10
The plot shown below are drawn using storage oscilloscope as our storage device. The first plot shows the charge collected in the muon pulses. This is achieved by calculating the area under the curve of the Paddle pulses. It is important to note that area under the curve does not actually give the charge,the value needs to be divided by unit of resistance to get the charge.
              Area under the   curve has units of  voltage-time. 
              Therefore charge collected= area/resistance
Signal from paddle is fed to CRO using cables. Now the characterictic impedance at the input of CRO is 50 ohm. So, if a current flows towards CRO, it would see an effective resistance of parallel combination ohf 1000ohm and 5o ohm, which is nearly 50 ohms.

Hence, charge collected=area/50 ohm

 The charge corresponding to each pulse height has been calculated and plotted. Moreover the histogram of the pulse height has also been plotted.
[image: image21.emf]Determination of charge  corresponding to muon pulses

0

0.002

0.004

0.006

0.008

0.01

0.012

0.014

0.016

0.018

0.02

-0.04 -0.035 -0.03 -0.025 -0.02 -0.015 -0.01 -0.005 0

Pulse height(V)

Charge(pC)


[image: image22.emf]Histogram of charge accumulated on PMT anode

0

20

40

60

80

100

120

140

0 20 40 60 80 100 120

Charge(10fC)

Counts


[image: image23.emf]Histogram of Pulse height of the Paddle pulses

0

1000

2000

3000

4000

5000

6000

7000

8000

0 5 10 15 20 25 30 35 40

Pulse height(mV)

Count

Total sample 11310


[image: image24.emf]Magnified scale histogram of pulse height

0

20

40

60

80

100

120

140

0 5 10 15 20 25 30 35 40

Pulse height(mV)

Count

Total sample 2114


So,now are convinced that RPC can be used to determine the charge collected due to  muon events on the paddle. Although, less number of fibers and attenuation play a spoil part but still our achievement upto this level is quite significant. 
Change over to Single fiber:

In this section, we proceed the same experiment with a single fiber i.e. instead of four fibers going to one PMT  we now have single fiber going to a single PMT. In this case we expect the pulse height to be very less as compared to the four fiber case!
Procedure:

· The paddle is taken out carefully and the tedlar sheet covering the fibers is opened.

· Three out of the four fibers are cut from the cookie ,but not completely removed ,for future use.

· Fiber is again carefully wrapped with the tedlar sheets and the paddle is again checked for light leakage and output. (We did not encounter any light leakage problem and output pulse is also well resolved from the noise)
A typical paddle pulse with a single fiber is shown below:

[image: image25.png]10.0ns 5.00GS/S & /-30.0mv
i+~ —4.80000ns J | 10k points

Assign
save [ 1O
waveform

save save Recall
Screen Image| waveform waveform





[image: image26.emf]Histrogram of ADC counts with Single Fiber

0

2000

4000

6000

8000

10000

12000

14000

16000

18000

10 20 30 40 50 60 70 80

ADC counts

frequency of the counts


[image: image27.emf]Magnified scale Histogram of ADC counts with Single fiber

0

200

400

600

800

1000

1200

1400

1600

1800

2000

8 1012141618202224262830323436384042444648505254565860

ADC counts

Frequency of the counts


But the histogram did not show us any signal peak!   Even the magnified scale histogram did not show any trace of any muon signal. This has happened eventually because of the long delay cables which reduce the initial amplitude by almost 58%. With single fiber ,already we expected the amplitude to be very very less and its attenuation due to the cables has made the situation worse. So we cant’t analyse the data obtained from the DAQ system and the ADC data and the experiment had to be halted unless we find out a solution to avoid the tremendous attenuation of the paddle signal. Maybe we can use external amplifiers to boost up the signal, but all this has not been done due to time constraint. We would greatly appreciate any help or suggestion regarding this problem and with the entire experiment as such.
References:

1) Radiation Detection and Measurement, by Glenn F. Knoll

2) Introduction to Nuclear Experimental Techniques, by W.R. Leo

3) Development and study of Glass Resistive Plate Chambers by Satyanarayan Bheesette
4) Introduction to Experimental Particle Physics by Richard Fernow
5) Nuclear Physics by SN Ghoshal

� EMBED Equation.3  ���





24..5cm


23.5cm


22.5cm








         1





         2





MAIN PADDLE


               3





               4





Operational voltage of PMT=1650V





Paddle pulse amplitude before delaying( 4 m  cable length)=24.4mV


Pulse amplitude after delaying(38m cable length)= 10.3mV


Percentage change in pulse height = 57.78%










































































_1307797822.unknown

